Setting Up the BitSURFER PRO EZ at 230400 on the LavaPort (ISA/PnP/PCI)

WINDOWS 95

1)Install the LavaPort as specified in the Manual provided leaving the baud rate at 115200.

2)Connect the BitSURFER PRO EZ to the LavaPort and install it using the Software provided by Motorola and configure the SPIDs appropriately.

3)Create a HyperTerminal connection for Direct to the LavaPort.

Ensure that the bps is set to 115200 or less.

4)Send the AT command to set the serial bps to 230400.  The command is as follows:

AT @P2=230400 <return>

5)Disconnect from HyperTerminal by either clicking the button with the receiver up or going to the Call menu and entering Disconnect.

6)Configure the Serial Port to 230400 by going to the File Properties menu and clicking the Configure button.

After setting the bps to 230400 click OK.

7)Reconnect to HyperTerminal by either clicking the button with the receiver down or going to the Call Menu and entering Connect.

8)Ensure that the serial bps for the TA is set for 230400 by sending the following command:

AT &v <return>

Look in the active profile for the @P2 setting.  If it does not equal 230400 then repeat step 4.

9)Save the Active Profile by sending the following command:

AT &W0 <return>

10)Exit HyperTerminal.

11)Add the new modem by deleting the .inf installed by the Motorola software and replacing it with the 230400 .inf provided and then rebooting.  In Windows 95 A this file is located in the Windows inf folder under the the filename oemX.inf where X is some integer.  In Windows 95 B (OSR2) the inf is located in the Windows inf Other folder under the filename Motorola.inf.

12)Add a new Dial Up Networking Connection ensuring that the bps is set

to 230400.